ECA - 5610 5G/6G Communication Systems

NAME : MOHIT V 192512043

EXP 6 5G NR Frame Structure Analysis with Different Subcarrier Spacing

Aim

To analyze the 5G NR frame structure by studying the effect of different subcarrier spacings,

slot durations, and OFDM symbols using MATLAB.

Objectives

1. To analyze different Subcarrier Spacing (SCS) values used in 5G NR numerology.

2. To compare the slot duration corresponding to different Subcarrier Spacing values in 5G

NR.

3. To study the number of OFDM symbols present in a 5G NR slot for normal cyclic prefix.

Software Required

• MATLAB

Theory

Unlike previous generations of mobile communication, 5G New Radio (NR) employs flexible

numerology, where different subcarrier spacings are used depending on the deployment scenario

and service requirements. Increasing the subcarrier spacing reduces the slot duration, enabling

lower latency and supporting diverse applications such as enhanced Mobile Broadband (eMBB),

Ultra-Reliable Low-Latency Communications (URLLC), and massive Machine-Type

Communications (mMTC). Regardless of the selected numerology, a normal cyclic prefix slot

contains 14 OFDM symbols.

Objective – 1

Analyze the standard subcarrier spacing (SCS) values defined in the 5G NR numerology using

MATLAB. Compare the relationship between the numerology index (μ) and subcarrier spacing to

understand the flexible bandwidth allocation employed in 5G communication systems.

Procedure

• Define the supported 5G NR subcarrier spacing values.

• Display the values using MATLAB.

• Plot the subcarrier spacing for different numerologies.

OUTPUT AND RESULT :

OBJECTIVE 1 :

OBJECTIVE 2 :

OBJECTIVE 3 :

Result:

The MATLAB analysis successfully demonstrated the flexible numerology of 5G NR. The results

showed that increasing the subcarrier spacing decreases the slot duration while maintaining a

constant number of OFDM symbols per slot under the normal cyclic prefix configuration.